### PT110N (10x PTZ Camera) A & E spec

Indoor Vandal Proof Pan/tilt/zoom (PTZ) dome camera with 10X zoom lens.

  - PT110N PTZ dome camera shall be manufactured by CBC Co., Ltd. whose quality system is in compliance with the ISO 9001, QUALITY SYSTEM.

  - The high-speed pan/tilt dome camera system shall meet or exceed the following design and performance specifications:

    - The system shall be microprocessor controlled with keypad-programmable nonvolatile memory.

    - Each pan/tilt drive system shall operate as an independent system with exclusive programming and setup data contained on each system’s nonvolatile memory.

    - The system shall feature an integral receiver/driver with Dip switch selectable addressing.

    - The system shall provide a built-in menu system for onscreen setup of camera functions.

    - The system shall be capable of 360-degree continuous pan rotation with a vertical tilt of -10 to 190 degrees.

    - The system shall pan under manual control from a creep speed up to 90 degrees per second.

    - The system shall operate under preset control at a maximum pan speed of up to 360 degrees per second and a tilt speed of up to 360 degrees per second.

    - The system shall be capable of up to 128 camera preset positions.

    - The system shall be capable of up to eight individual sequence group tours.

    - The system shall be capable of four individual  programmed pattern tour.

    - When an operator stops manual control of the camera, and a programmed period

of time is allowed to expire, the camera will execute one of the following programmable functions: 1) return to preset or 2) return to auto swing 3) return to auto group or 4) return to pattern.

    - The system shall feature zone titling.

    - The system shall provide -10 - +190 tilt degree camera view

    - The system shall provide four programmable automatic swing.

    - The system shall provide auto flip feature that automatically pans the camera 180 degrees when tilted fully down to follow an object, moving directly underneath the dome.

    - The system shall be available in NTSC video signal format.

    - The system shall provide a composite video output of 1.0 V p-p.

    - The image capturing device shall be a 1/4-inch IT CCD.

      - The image sensor shall have a total pixel array of:

        - 380,000 (NTSC)

    - The system shall provide a horizontal resolution of:

      - 500 TV lines, NTSC

    - The scanning system shall be 2:1 interlacing.

    - The system shall have digital signal processing.

    - The system shall have automatic white balance.

    - The system shall have automatic iris control and manual override.

    - The system shall provide backlight compensation.

    - The system shall have digital slow shutter function.

    - The system shall have mechanical Day & Night function.

    - The system shall provide electronic shutter speeds of:

      - 2 to 1/120,000 sec  NTSC

    - The system shall provide a signal-to-noise ratio of 50 dB.

    - The system shall have the following sensitivity specifications:

      - 0.7 lux Color, 0.02lux B/W

    - The system shall have a privacy masking feature.

      - The user shall be able to set up to 4 privacy masks.

  - The motorized lens shall meet or exceed the following design and performance specifications:

    - The camera shall be equipped with a 10X optical zoom lens(focal length : 3.8 mm to 38 mm

Horizontal angle of view  51.2-5.58 degree)

    - The lens shall feature an additional 10X programmable digital zoom for a total of at least 100X.

    - The lens shall feature an automatic focus with manual override.

  - The system shall have the following additional specifications:

    - Electrical

      - Input voltage: 24 VAC at 50 Hz for NTSC

      - Input power: 400 mA

    - Control signal input

      - RS-485

    - Accessories

      - The camera shall be available in wall mount, pendant mount, and flush mounted versions. Proper mounting hardware kits shall be provided according to the model specified.

    - The camera shall provide four normally open or normally closed alarm input contacts and outputs.

    - Environmental

      - The Indoor system shall be vandal resistant.

  - The camera and system shall conform to these internationally recognized compliance standards:

    - FCC