ARCHITECTURAL AND ENGINEERING SPECIFICATIONS

Section 16780 – Video Surveillance Systems

ZC-D8000B Series (ZC-D8312NBA, Z-DN8312NBA, ZC-D8039NBA, ZC-DN8039NBA)

(ZC-D8312PBA, Z-DN8312PBA, ZC-D8039PBA, ZC-DN8039PBA)

Color, True or Digital Day/Night, Fixed Mini Dome, High Resolution, Surveillance Camera

PART 2 – PRODUCTS

2.01     MANUFACTURER

A.  CBC Co. Ltd.

2-15-13, Tsukishima

Chuo Ku, Tokyo, Japan

Tel:  81(3) 3536-4840

Fax: 81(3) 3536-4840

B.	This product shall be manufactured by a firm whose quality system is in compliance with the ISO 9001, QUALITY SYSTEM.

2.02      GENERAL REQUIREMENTS

Color, True or Digital Day Night, Indoor, Fixed Dome Camera

A.	The camera specified should be a full-featured by advanced OSD control, fixed dome camera designed for indoor applications. The camera shall be a 600 TVL resolution, high sensitivity integral color camera using a 1/3-inch format CCD imager, equipped with IR cut filter removal for TDN models and prepackaged in a housing with a clear optical grade plastic dome. As required by the application, the camera shall come equipped with an integrated

3.3 to12mm,F1.4; 3 to 9mm, F1.2 DC auto iris lens.

B.    The optical system shall permit the camera to cover a large angle from 89.8 to 23.9 Horizontal (3.3-12mm) or 90.0 to 31.8 Horizontal (3- 9mm). The lens Zoom and Focus levers shall have locking screws. A level adjustment potentiometer shall be available to adjust the iris for optimum exposure.

C.    The camera shall require a minimum scene illumination of 0.4lux at F1.4 (50IRE, Digital DN, AGC ON) or 0.04lux at F1.4 (50IRE, True DN, AGC ON) at nigh

D.  The camera shall utilize Digital Signal Processing (DSP) to provide high stability and reliability

E.   The camera shall have an optional module for video transmission via UTP (Unshielded Twisted Pair) cable.

F.	The manufacturer shall offer models of the camera for use in NTSC (60Hz) / PAL(50Hz) CCTV systems. The cameras shall operate within the voltage range of 24VAC or 12VDC.

G. The camera shall have 2D Digital Noise Reduction function.

H.   The camera shall have HLC (High Light Compensation) function

I.   The camera shall have Contrast Adjustment function

J.    The camera shall have easy focus adjustment function.

K.   The camera shall have Privacy masking function.

L.   The camera shall have Motion detection function.

M.    The camera shall have Camera title function.

N.   The camera shall have MIRROR function.

O.  The camera shall have Digital WDR function.

P.   The camera shall have negative image output function.

Q.	The manufacturer shall offer the proper hardware accessories to allow the specified camera to be wall-mounted, flush-mounted, surface-mounted or pendant-mounted, as required by the application.

R.  For ease of on-site setup and adjustments once the dome camera is installed, the product shall provide, in addition to the standard video output BNC connector, a service video monitor output connector. This connector shall provide a composite video output signal.

2.03     VIDEO ELECTRICAL REQUIREMENT

A. Input voltage range: either 12VDC 10% or 24VAC10% for power

B. Composite Video Output: 1.0 V peak to peak, 75 ohms.

C. Scanning System: 2:1 Interlace NTSC (525 lines, 60 fields/30 frames)

PAL (625 lines, 50 fields/25 frames)

D. Sync: Internal Sync.

E. Signal System: NTSC (60Hz) PAL(50Hz) standard.

F. Resolution: Horizontal: More than 600 TVL.

G. Imager: 1/3-inch, interline transfer CCD

H. Signal to noise ratio: More than 50 dB.

I. White Balance: Auto Tracking White (ATW) mode.

J. Backlight compensation: Internally selectable On/Off.

K. Power Consumption: 2.0W at 12VDC, 2.4W at 24VAC

2.04 MECHANICAL SPECIFICATIONS:

A. The camera shall come with slotted, plastic black insert, installed onto the main camera section, such that the lens will only be visible when viewed through that slot.

B. The lower dome cover shall be made of optical grade plastic material

C. Connectors: Video Out: Female BNC on coax.

D. Internal video monitor connector.

E. The main camera body shall have a 3 axis gimbals mount for wall or ceiling mountings.

F. The camera shall weigh 280g

G.  Dimensions of the camera shall be 4.7” (φ) x 4.3” (H), 122mm (φ) x 108.3mm (H).

2.05

ENVIRONMENTAL SPECIFICATIONS:

A. Temperature:

1) Operating: -10C to +50C (14F to +122F)

2) Storage: -20C to +60C (-4F to +140F)

B. Humidity: 0% to 85% relative, non-condensing.

2.06 SUPPLIED ACCESSORIES

A. Operating Instructions

B. Mounting Template

2.07 OPTIONAL ACCESSORIES

A. NVT Transmitter for use with CAT5 cable

B. Flush Mount Kit

C. Pendant Mount Adapter

D. Wall Mount Kit

E. Tinted Dome Cover

F. Single Gang Box Mounting Plate

2.08 REGULATIONS

A. Electromagnetic Compatibility: FCC class A

B. CE

C.WEEE

D. UL