ARCHITECTURAL AND ENGINEERING SPECIFICATIONS

Section 16780 – Video Surveillance Systems

ZC-D5000 WDR Series (ZC-DW5212NXA, ZC-DW5550NXA, ZC-DWN5212NXA)

WDR, Mechanical or Digital Day/Night, Fixed Mini Dome, High Resolution, Surveillance Camera

PART 2 – PRODUCTS

2.01	MANUFACTURER

A. CBC (AMERICA) Corp.

55 Mall Drive, Commack

NY 11725

Tel. (800) 422-6707

Fax. (631) 543-5426

B.	This product shall be manufactured by a firm whose quality system is in compliance with the ISO 9001, QUALITY SYSTEM.

2.02	General Requirements

WDR, Mechanical or Digital Day Night, Indoor, Fixed Dome Camera

A.	The camera specified should be a full-featured by advanced OSD control, fixed dome camera designed for indoor applications. The camera shall be a 700 TVL resolution, high sensitivity integral color camera using a 1/3-inch format CCD imager, equipped with IR cut filter removal and prepackaged in a housing with a clear optical grade plastic dome. As required by the application, the camera shall come equipped with an integrated CS Mount 2.8 to 12mm, F1.3; 5 to 50mm, F1.3 DC auto iris ens.

B.    The optical system shall permit the camera to cover a large angle from 98.2 to 28.3 Horizontal (2.8-12mm) or 51.8 to 5.6 Horizontal (5- 50mm). The lens Zoom and Focus levers shall have locking screws. A level adjustment potentiometer shall be available to adjust the iris for optimum exposure.

C. 	The camera shall require a minimum scene illumination of 0.3lux at F1.3 (50IRE,Digital DN,AGC ON) or 0.08lux at F1.3 (50IRE, True DN, AGC ON) in color mode.

The camera shall require a minimum scene illumination of 0.03lux at F1.3 (50IRE,Digital DN,AGC ON) or 0.008lux at F1.3 (50IRE, True DN, AGC ON) at night.

D.    The camera shall require a minimum scene illumination of 0.00006lux at F1.3 (50IRE,Digital DN,AGC ON) or 0.000016lux at F1.3 (50IRE, True DN, AGC ON) in sense up mode.

E.   The camera shall utilize Digital Signal Processing (DSP) to provide max 82dB (12,600times)

Wide Dynamic Range.

F. The camera shall have an optional module for video transmission via UTP (Unshielded Twisted Pair) cable.

G	The manufacturer shall offer models of the camera for use in NTSC (60Hz) CCTV systems. The cameras shall operate within the voltage range of 24VAC or 12VDC.

H. The camera shall have 2D/3D Digital Noise Reduction function.

I.   The camera shall have HLC (High Light Cut) Function

J.   The camera shall have FBC (Fine Black Correction) Function

K.   The camera shall have 4 patterns of Profile Setting.

L.    The camera shall have Easy focus function.

M.   The camera shall have Privacy masking function.

N.   The camera shall have Motion detection function.

O.    The camera shall have Camera title function.

P.   The camera shall have Flip/Rotate function.

Q.   The camera shall have Test pattern output function.

I.	The manufacturer shall offer the proper hardware accessories to allow the specified camera to be wall mounted, flush mounted, surface mounted or pendant mounted, as required by the application.

J.  For ease of on-site setup and adjustments once the dome camera is installed, the product shall provide, in addition to the standard video output BNC connector, a service video monitor cable. This connection shall provide a composite video output signal.

2.03     Video Electrical Requirement

A. Input voltage range: either 12VDC 10% or 24VAC10% for power

B. Composite Video Output: 1.0 V peak to peak, 75 ohms.

C. Scanning System: 525 lines, 60 fields/30 frames, 2:1 interlace

D. Signal System: NTSC (60Hz) standard.

E. WDR : Max 82dB (12,600x)

F. Resolution: Horizontal: More than 700 TVL.

G. Imager: 1/3-inch, interline transfer CCD

H. Signal to noise ratio: More than 50 dB.

I. White Balance: Auto Tracking White (ATW) mode.

J. Backlight compensation: Internally selectable On/Off.

K. Power Consumption: 230mA at 12VDC, 3.0W at 24VAC

2.04 MECHANICAL SPECIFICATIONS:

A. The camera shall come with slotted, plastic black insert, installed onto the main camera section, such that the lens will only be visible when viewed through that slot.

B. The lower dome cover shall be made of optical grade plastic material and have a removable safety strap attached to the main body for safety.

C. Connectors: Video Out: Female BNC on coax.

D. Internal video monitor connector: Detachable monitor cable with Female BNC.

E. The main camera body shall have a 3 axis gimbals mount for wall or ceiling mountings. Adjustment angle is 350 Pan; 180 Tilt; 350 Rotate

F. The camera shall weigh between 470g (14.1oz) and 560g (19.8oz) depending on the lens.

G.  Dimensions of the camera shall be 5.7” (W) x 4.3” (H), 144mm (W) x 110mm (H).

2.05 ENVIRONMENTAL SPECIFICATIONS:

A. Temperature:

1) Operating: -10C to +50C (14F to +122F)

2) Storage: -20C to +60C (-4F to +140F)

B. Humidity: 0% to 85% relative, non-condensing.

2.06 SUPPLIED ACCESSORIES

A. Operating Instructions

B. Service Monitor Cable

C. Mounting Template

2.07 OPTIONAL ACCESSORIES

A. NVT Transmitter for use with CAT5 cable

B. Flush Mount Kit

C. Pendant Mount Adapter

D. Wall Mount Kit

E. Tinted Dome Cover

F. Single Gang Box Mounting Plate

2.08 REGULATIONS

A. Electromagnetic Compatibility: FCC class B.