ARCHITECTURAL AND ENGINEERING SPECIFICATIONS

Section 16780 – Video Surveillance Systems

ZN-C Series (ZN-C1, ZN-C1M, ZN-C2M)

Network, C/CS mount Mega-Pixel / VGA Color Camera, True Day/Night, Surveillance

PART 2 – PRODUCTS

2.01  MANUFACTURER

A.  CBC Co. Ltd.

2-15-13, Tsukishima

Chuo Ku, Tokyo, Japan 104-0052

Tel:  81(3) 3536-4840

Fax: 81(3) 3536-4840

B.	 This product shall be manufactured by a firm whose quality system is in compliance                   with the ISO 9001, QUALITY SYSTEM.

  - GENERAL REQUIREMENTS                                                                                                            A. General Characteristics:

1. The camera is designed to provide advanced image support for resolutions up to                                                            1080p (ZN-C2M), 720p (ZN-C1M), or VGA (ZN-C1).

2. The camera provides dual-codec functionality, for simultaneous transmission of H.264                                                       and Motion JPEG (MJPEG) video.

3. The camera is designed to provide H.264 compressed video of up to 1080p (ZN-C2M),                                                    720p (ZN-C1M) or VGA (ZN-C1) res. at image rates up to 30 images per second (ips).

4. The camera uses a high-quality 1/2.5” Progressive Scan CMOS image sensor.

5. The camera can be assembled with  C/CS mount lens with DC Auto-Iris functionality.

6. The camera shall provide Color pictures in light as low as 0.7 lux @F1.2  30 IRE(ZN-C1),                                                       0.8 lux @1.2 30 IRE ( ZN-C1M,  ZN-C2M )

7. The camera shall provide B&W pictures in light as low as 0.06 lux @ F1.2    Slow Shutter ON                                               30 IRE( ZN-C1, ZNC1M, ZN-C2M)

8. The camera shall be capable of accepting a power source from either 12VDC, 24VAC or                                                 IEEE 802.3af  Power-over-Ethernet (PoE).

9. The camera shall be capable of transmitting analog PAL or NTSC (1Vp-p) video                                                                     for setup and installation purposes.

B. Adjustments and features:

1. Progressive Scan CMOS technology for optimal image clarity.

2.  Resolution can be selectable as follows.

ZN-C2M  :    1920 x 1080 pixels,  1280 x 720, 1280 x 720(wide), 1280 x 1024, 1024 x 768,                                                                                                                                                  704 x 576, 704 x 480, 640 x 480, 352 x 288, 352 x 240, 320 x240

ZN-C1M   :  1280 x 720 pixels, 1280 x 720(wide), 1280 x 1024, 1024 x 768, 704 x 480, 704 x 576,                                               640 x 480, 352 x 288, 352 x 240, 320 x 240

ZN-C1    :    704 x 576 pixels, 704 x 480, 640 x 480, 352 x 288, 352 x 240, 320 x 240

3. Provides True Day/Night functionality, allowing automatic switching between Color mode and                                               B&W mode, as well as removal of an IR cut filter for enhanced low light performance.

4. Camera shall have integrated, programmable motion detection, with up to four (4) user-defined                                   detection zones, and the ability to map each detection zone to be able to trigger a recording                                            event and/or analog alarm output from the camera unit.  Each zone can be configured                                                       with a 10-level sensitivity setting.

5. The camera shall support Back Light Compensation (BLC).

6. Two (2) authority levels of user privileges shall be available:

a. Admin level shall have full access to all functions.                                                                                                                                                                                                                                      b. User level will be restricted to monitoring live video only.

2.03  VIDEO ELECTRICAL REQUIREMENT

A. Imager: Progressive Scan 1/2.5” CMOS Sensor.

B. Input voltage range : 12VDC plus and minus 10% or 24VAC plus and minus 10% or PoE

C. Composite Video Output for service monitor (installation mode only) : 1.0V peak to peak, 　　　　　　　　　　　　75 ohms.  NTSC/PAL selectable

D. Shutter: Electronic range (in seconds) of  1/4, 1/10, 1/12.5, 1/15, 1/20, 1/25, 1/30, 1/50, 1/60,                                         1/100, 1/120, 1/240, 1/480, 1/960, 1/1024, 1/2000.

E. Minimum illumination :                                                                                                                                                         Color picture                  0.7 lux @F1.2  30 IRE(ZN-C1),0.8 lux @1.2 30 IRE ( ZN-C1M,  ZN-C2M )                                  Black & White picture    0.06 lux @ F1.2 Slow Shutter ON 30 IRE( ZN-C1, ZNC1M, ZN-C2M)

F.  White Balance  : Auto-tracing White (ATW) mode, and Manual control with Preset mode.

G.  Backlight Compensation (BLC) : supported.

H.  Electronic iris : Selectable On/Off.

I..  Day/Night : Controlled by IR cut filter IN/OUT function     Selectable Auto/Color/Black & White.

J.  AGC Gain: Selectable On/Off.  Adjustable gain control in ‘Off’ mode.

K.  Power Consumption

ZN-C2M   :  397mA, 4.8W @ 12VDC       328mA, 4.7W @ 24VAC      91.7mA  4.4W Class 2 @PoE                                              ZN-C1M   :  363mA, 4.4W @ 12VDC       319mA, 4.3W @ 24VAC      87.2mA  4.2W Class 2 @PoE                                                  ZN-C1      :  353mA, 4.2W @ 12VDC       305mA, 4.2W @ 24VAC      85.1mA  4.1W Class 2 @PoE

2.04  OPERATING ENVIRONMENT

A. PC: IBM PC/AT or compatible.

B. Operating Systems: Windows XP Professional (32-bit), Windows Vista, Windows 7.

C. CPU: Pentium (or equivalent) 2.80GHz (or higher).

D. Memory: 1GB (or higher).

E. Network Interface: 10/100/1000 Base-Tx (up to 1Gigabit operation) (RJ-45 connector).

F. Display Card: 1280 x 1024 minimum resolution; DirectX version 8.0 (or higher).

G. Graphics Chip: NVIDIA GeForce 8500GT 256MB (or higher).

H. Web Browser: Internet Explorer 7.0 (or higher).

2.05  NETWORK  INTERFACES

A. Protocols supported:

IPv4/6, HTTP, SMTP, UPnP, DNS, DDNS, NTP, RTSP, RTP, TCP, UDP, IGMP, RTCP, ICMP, 　　　　　　　　　DHCP, ARP.

B. Camera IP interface: RJ-45 10/100 Base-Tx, PoE compatible.

C. The camera shall interface to:

Microsoft Internet Explorer 7.0, or above.

Compatible PDA / smart phone

D. When accessed from a browser online, the camera shall provide a web-based graphical user interface 　　　　　　　and menu system,   for viewing and configuration of the camera.

E. Web access protected by user ID and password.                                                                                                                    The camera shall maintain an event log which includes all remote login events.

F. Up to 4 users may access a camera via the web server simultaneously.

G. One (1) alarm input (NO/NC) shall be provided.

H. One (1) alarm output (NO/NC) shall be provided.

I. One (1) audio input (mono, line-level, unbalanced) shall be provided.

J. The camera shall support both static IP addresses and dynamically-assigned IP addresses provided 　　　　　　　　　by a Dynamic Host Control Protocol (DHCP) server.

K. Third party application integration of the camera is supported via HTML Application Programming 　　　　　　　Interface (API), which would provide the necessary information and command lines for accessing 　　　　　　　　　the camera using command lines or     third-party software application.

L.  This device is certified ONVIF compliant (conformant), allowing it to be accessed using a generic 　　　　　　　　ONVIF device driver.

M. This device has the ability to upload still snapshot JPEG images to an FTP site, based on                                                 user-defined events or regularly upon scheduled intervals.

N. This device supports Micro-SDHC cards from most major brands, capacity up to 32GB.

2.06 . MECHANICAL SPECIFICATION

A. Weight: 250g    (0.552 lbs.)

B. Dimensions: 64.6mm x 60.3mm x 82.7 / 2.5” x 2.4” x 3.2”

2.07  ENVIRONMENTAL SPECIFICATION

A. Operating Temperature :  -10ºC ~ 50ºC (14°F ~ 122°F)    [ not cold start ]

B. Humidity                        :   less than 90%, non-condensing

2.08  ELECTROMAGNETIC COMPATIBILITY                                                                                                                                 CE, FCC, UL,

2.09  REGULATION                                                                                                                                                                            RoHS, WEEE