GENERAL – NOT USED

PRODUCTS

HIGH DEFINITION CAMERA WITH EMBEDDED VIDEO ANALYTICS

Manufacturer:

CBC Co. Ltd.
2-15-13, Tsukishima Chuo Ku Tokyo Japan 104-0052                      
Tel : 81(3) 3536-4840 Fax : 81(3) 3536-4840

General

The camera shall:

Be designed to deliver embedded video analytics.

Be designed to provide H.264 and Motion JPEG (MJPEG) video.

Be designed to support resolutions up to 1920 x 1080 pixels.

Be designed to provide video at 30 frames per second (fps) for all resolutions.

Provide an embedded web-server.

Be equipped with a slot for microSD/SDHC memory card.

Be equipped with built-in IR LEDs.

Support Power over Ethernet (PoE) as a power source.

Be designed to meet IP66 level of dust and water proofed.

The camera mount accessaries available shall include:

Pole

Junction Box

Hardware

The camera shall meet or exceed the following specifications:

1/2.7" progressive scan CMOS sensor

Mechanical IR-cut filter

DC auto iris lens

3x optical zoom lens

3.0 mm (wide) and 9.0 mm (tele)

F1.2 (wide) and F2.1 (tele)

Motorized Zoom and Focus (MFZ)

Angle of view

Horizontal: Between 93° and 31.7°

Vertical: Between 68.4° and 23.8°

Minimum Illumination

IR Sensitivity Range: 700-1100 nm

Day mode (Color): 0.5lux at F1.2 (with IR-filter in use)

Night mode (Black and White): 0lux at F1.2 (with IR-filter removed, IR Illuminator On)

IR Illuminator

IR LED: IR LED x 34 (850 nm)

IR working distance: 25m (0 lux, 30IRE, Max Exposure Gain)

The camera shall automatically adjust the level of the infrared light to its best, providing so-called Smart IR

Audio: 1 x Audio In, 1 x Audio Out

External I/O Terminals: 1 x Alarm In, 1 x Alarm Out

Analogue Video output for installation

256MB Flash memory and 256MB RAM

microSD/SDHC memory card slot

The camera shall support up to 64GB SD memory card.

Video

Supported Encording format shall include:

H.264 Baseline, Main, High profile (MPEG-4 Part 10/ AVC)

MJPEG (Motion JPEG)

Video Streaming shall provide:

Two separate video streams which are individually configurable (e.g. Alarm event at high quality and Continuous recording at low quality)

Configurable range of 1 ~ 30 fps in all resolutions of MJPEG

Configurable range of 1 ~ 30 fps in all resolutions of H.264

Configurable Group of Pictures (GOP) in H.264

Constant Bit Rate (CBR) and Variable Bit Rate (VBR) in H.264

Configurable image quality (Highest, High, Normal, Low, Lowest) in VBR mode of H.264

Configurable JPEG quality in MJPEG

Supported video resolution shall include:

320x180

480x270

640x360

800x450

960x540

1120x630

1280x720

1920x1080

Image control shall include:

Brightness, Contrast, Saturation and Sharpness

Image rotation (Vertical flip, Horizontal mirror)

Electronic shutter speed (Automatic and Manual, 1/2 ~ 1/5000)

Auto Gain Control (AGC)

Exposure adjustment (-1.0, -0.6, -0.3, 0, +0.3, +0.6, +1.0 EV)

Back Light Compensation (BLC)

IR Illuminator Control (Smart IR, Manual)

Digital Slow Shutter (DSS)

Wide Dynamic Range (WDR)

Automatic and Manual Day (color) and Night (black and white)

Automatic and Manual White Balance

2D Digital Noise Reduction (DNR)

Audio

The camera shall meet or exceed the following specifications:

Two-way full duplex audio

Input sources

External microphone

External line device

Output sources

External line device

Encoding

G.711 uLaw at 8/16kHz

Video Content analytics (VCA)

General

The VCA software shall be embedded in the camera, so it can keep the latency of the alarm at minimum.

The video source used for the VCA software shall not be affected by any encoding/decoding actions performed by Video Management Software (VMS), Digital Video Recorder (DVR) and Network Video Recorder (NVR) before the VCA is performed.

The embedded VCA software shall be ready to use right out of the box minimizing difficulties in installation and maintenance.

The VCA software shall be usable for both indoor and outdoor video environment.

The VCA software shall operate with various video sources including color, black and white, SD resolution, HD resolution.

The VCA software shall provide a variety of detection zones. A detection zone is defined as a dedicated region within a camera’s field of view used to detect behaviors specific to that zone.

Multiple zones may be defined in a single camera view. The camera shall provide at least 40 individually configurable zones.

The VCA software shall provide a variety of detection rules. A detection rule is defined as a dedicated filter applied to a detection zone characterizing a specific behavior to detect for an object being tracked.

The VCA software shall continually track moving and stationary targets and generate real-time alerts of object presence in multiple overlapping detection zones.

Multiple rules may be applied in a single detection zone. The camera shall provide at least 60 simultaneously operable rules.

The VCA software shall be capable of detecting and tracking up to 100 objects simultaneously.

The VCA software shall provide calculated size and speed of tracked objects with an additional calibration. The VCA software shall provide 3 dimensional virtual grid, ruler and human figure to reduce time and effort for the calibration.

The camera shall provide Metadata in plain XML format for third party applications.

Detection zones shall include:

Non-detection zone

The VCA software shall provide a special zone that will suppress alarm generation until an object has left the object blocking zone. Object will be tracked while it is in the zone, but this will not generate alarms till it leaves the zone. This can be used in areas such as wildly moving trees, reflective surfaces, or moving door and gates and will greatly reduce the number of false alarms.

Line

Polygon

Detection rules shall include:

Enter/Exit/Appear/Disappear filter

An object entered alarm is raised when an object crosses from the outside to the inside of a detection zone. Conversely, an object exited alarm is raised when an object crosses from the inside to the outside of a detection zone.

Stopping filter

Objects that are stopped inside a detection zone for longer than the defined amount of time will trigger the detection rule and raise an alarm.

Dwell filter

Objects that dwell inside a detection zone for longer than the defined amount of time will trigger the detection rule and raise an alarm.

Direction filter

Objects that travel in the configured direction (within the limits of the acceptance angle) through a detection zone, trigger the detection rule and raise an alarm.

Tailgating filter

Object tailgating is defined as an object crossing a detection zone within a certain time after an object has already crossed the zone.

Object Classification filter

Object classification filter can be activated once the camera is calibrated. The object classification is based on properties extracted from the object including object area and speed.

Speed filter

Objects that travel within the bounds of the configured speeds, through a detection zone trigger the detection rule and raise an alarm.

The camera shall provide:

Tamper Detection which shall detect camera tampering events such as bagging, de-focusing, moving the camera, etc. This is achieved by detecting large persistent changes in the image.

At least 20 Counters which count triggers generated by detection rule violation. For example, if it is required to count the number of objects entering a detection zone, the zone must initially be configured to raise an alarm every time an object enters it. The zone can then be assigned to a counter and the counter will count the objects according to the type of counting required. Supported types of counting are:

Increment

Decrement

Occupancy

Counting Line which is specifically designed as a detection filter optimized for bi-directional object counting in busier detection scenarios.

Networking

The camera shall connect to the network via a RJ-45 with built-in Auto switching 10/100 Mbit/s Ethernet interface.

The camera shall support fixed IP addresses

The camera shall support IP addresses dynamically obtained by a Dynamic Host Control Protocol (DHCP)

IP addresses shall be compliance with the IP version 4 (IPv4)

Supported protocols shall include:

QoS Layer 3 DiffServ, TCP/IP, UDP/IP, HTTP, HTTPS, RTSP, RTCP, RTP/UDP, RTP/TCP, mDNS, UPnP™, SMTP, DHCP, DNS, DynDNS, NTP, SNMPv1/v2c/v3(MIB-II), IGMP, ICMP, SSLv2/v3, TLSv1

Video streaming protocol shall include:

HTTP (Unicast)

HTTPS (Unicast)

RTP over RTSP (Unicast & Multicast, UDP & TCP)

Video streaming protocol shall:

Provide Automatic and Manual Bandwidth control

Provide Selection for components of video stream (audio and metadata) to reduce required bandwidth

Support Quality of Service (QoS) to be able to prioritize network traffic for video, audio and metadata

Web interface

Web interface shall provide:

Live view

Local storage management (SD/SDHC card)

Configuration page for the camera

ActiveX software installation for specific task

ActiveX software shall:

Be downloaded directly from the camera

Display live video images from the camera

Recording

The camera shall record and store videos into a SD memory card mounted in the camera and into an external storage server, such as FTP server.

All video recording files shall be listed and downloaded via web interface.

The recording in SD memory card shall be instantly started for a pre-defined timeframe by request of the user via the web interface, providing so-called Instant Recording.

Continuous Recording

The camera shall support continuous video recording in SD memory card.

The camera shall automatically start replacing old video footages with new video recordings when there is not enough space left in the SD memory card.

The camera shall allow video recordings to be stored in SD memory card being segmented by pre-defined length or by pre-defined size.

Event Recording

The camera shall support Event alarm based Recording in SD memory card and in an external FTP server.

The camera shall provide at least 5 seconds of pre alarm recording and 60 seconds of post alarm recording.

The camera shall provide a search interface for recording files in SD memory card, allowing the files to be searched by a specific event with a given time period.

The camera shall ensure a reliability of video file transfer into the external FTP server against any incident, such as connection to camera is down or recording FTP server is down. This is done by utilizing SD memory card as a buffer. It will resume the video file transfer after a recovery from system or network failure, providing so-called fail over recording on SD card.

Event Management

Event shall be triggered by:

External Sensor (DI, Digital Input) which shall programmatically work as a normally open type sensor or a normally close type sensor

Motion Detection (MD)

Video Content Analytics (VCA)

Network configuration change

Fan/Heater operation change

When an event triggered, it shall:

Activate an external alarming device (DO, Digital Output)

Start recording in SD memory card and in an external FTP server

Send a notification message with a snapshot via Email

Send HTTP notification

Send TCP notification

Save a notification message and a snapshot in an external FTP server

The camera shall provide a search interface for events, allowing the events to be searched by a specific event type with a given time period.  A video recording file for the event, if it is available, shall be downloaded via the search interface.

The camera shall provide a functionality of automatic and manual event log transfer to an external FTP server.

Text Overlay

Text Overlay is defined as a function which delivers on-screen embedded texts or drawings over a video stream and a snapshot. Supported element of the Text Overlay shall include:

Burnt-in Text

Burnt-in VCA annotation

Privacy mask

The Burnt-in Text shall:

Deliver a customer-specific text of at least 48 ASCII characters.

Deliver date and time

The Burnt-in VCA annotation shall deliver:

Detection zone and line

Non-detection zone

Object tracking bounding box

Counter

Object classification

Object size and speed

The camera shall provide at least 4 individually configurable privacy masks to conceal defined areas in the image as non-viewable.

Application Programmable Interface (API) support

The camera shall be fully supported by an open API, which shall provide necessary information for integration of funcionality into third party applications.

Supported Third Party API shall include:

ONVIF Profile S

GENETEC Protocol

Security

The camera shall:

Support the use of HTTPS and SSL/TLS

Provide multiple user account with a password protection restricting access to the built-in web interface and video stream

Provide authentication procedure which requires users to view video stream using an account ID and a password. The ID and password shall be encrypted by the Digest method (MD5) before being transferred.

Maintenance

The camera shall:

Be supplied with MS Windows-based management software, which discovers the cameras in the same network and allows assignment of IP addresses, firmware update and rebooting the camera.

Allow firmware (FW) update over the network via the web interface.

Allow backup system logs into an external File Transfer Protocol (FTP) server

Allow backup and reload a user-specific configuration data via the web interface.

Diagnostics

The camera shall:

Be equipped with an LED, indicating the camera’s functional status.

Be equipped with a built-in temperature sensor, which measures the internal temperature of the camera. This sensor shall be able to turn on a built-in heater or a built-in blower if the temperature is outside of the official operative range.

Be monitored by a Watchdog functionality, which shall automatically re-initiate processes, restart the unit if a malfunction is detected or turn on Safe mode providing a simple interface to upload Firmware (FW) if Operating System (OS) is damaged.

Provide a heart beat signal, which continuously transfers a signal over network to a pre-defined destination device with a certain time interval. It may be an indicator of which ensures whether or not the camera is alive.

Provide system log file which shall keep at least 10000 records. The camera shall keep records in log file when:

Any event occurs

Any event configuration is changed

Network configuration is changed

CPU is overloaded

System memory is overused

Environmental

The camera shall meet or exceed the following specifications:

Weatherproof: Rated IP66 or higher

Operating Temperature Range

DC 12V: -40 to 122 degrees Fahrenheit (-40 to 50 degrees Celsius)

PoE: 32 to 122 degrees Fahrenheit (0 to 50 degrees Celsius)

Cold Start Temperature

DC 12V: -4 degrees Fahrenheit (-20 degrees Celsius)

PoE: 32 degrees Fahrenheit (0 degrees Celsius)

Relative Humidity Range

Up to 85%, non-condensing

Built-in heater

Power Requirement

DC 12V

Input voltage range: 10.8 ~ 13.2 VDC

Consumption: max. 14.88 Watt

Power Over Ethernet

Standard: Class 0 (IEEE 802.3af)

Warranty

Manufacturer shall provide at least a 3 years warranty on parts and repair labor for the camera commencing with the date of purchase.

MANUFACTURED UNITS

The camera shall be:

Ganz ZN-BT662XE-MIR Indoor/Outdoor Remote Zoom and Focus IR Bullet Network Camera

Dimensions: 3.3 inches (83mm) width and 3 inches (75.7mm) length by 12.7 inches (322mm) high

Color: Ivory

Weight: 3.3 pounds (1.5kg)

Provided materials shall include:

Installation Guide

User’s Guide

API Guide

Mounting template

EXECUTION – NOT USED

END OF SECTION